# SIMATS ENGINEERING

Department of Computer Science & Engineering

## ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

### ANNEXURE A

#### Constraint-Based Problem Solving

- Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

  - CPU utilization must be below 15%

  - Multiple sensors generate interrupts every 1 ms

  - Use vectored interrupt handling

  - Select between memory-mapped and I/O-mapped I/O for optimal latency

- Design an I/O subsystem under these constraints:

  - High-speed storage requires throughput > 3 GB/s

  - Must use NVMe over PCIe

  - CPU should not handle bulk data transfer

  - Integrate DMA controller with interrupt notification

- Construct a multi-device communication architecture with constraints:

  - 6 peripherals using USB, 2 high-speed devices using Thunderbolt

  - Avoid interrupt starvation

  - Use hardware and software interrupts

  - Ensure fair bus arbitration

- Design a data acquisition system with constraints:

  - Continuous data stream at 500 MB/s

  - Minimal CPU intervention required

  - Must compare DMA transfer vs interrupt-driven I/O

  - Use memory-mapped I/O for device registers

- Construct an interrupt handling mechanism with constraints:

  - Support nested interrupts

  - Maximum interrupt latency < 10 µs

  - Use vectored interrupts for high-priority devices

  - Use software interrupts for background tasks

#### Code of Conduct:

I _Jasim Asmaan A 192421060_ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

### MARKS ALLOCATION

|  | Problem Understanding (25%) | Constraint Analysis (25%) | Solution Development (25%) | Application of Concepts (15%) | Justification (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

### RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |

QUESTION 1: REAL-TIME EMBEDDED I/O COMMUNICATION MECHANISM

Given Constraints

CPU utilization must remain below 15%.

Multiple sensors generate interrupts every 1 ms.

Vectored interrupt handling must be used.

A suitable I/O communication method must be selected for low latency.

Proposed Solution

A vectored interrupt controller is used to manage the interrupt requests generated by multiple sensors. Each sensor is assigned a separate interrupt vector. When a sensor generates an interrupt, the interrupt controller identifies the source and directs the CPU to the corresponding Interrupt Service Routine (ISR).

Memory-mapped I/O is selected because the sensor registers can be accessed directly using normal memory instructions. This provides simple programming and fast access to sensor data.

To maintain CPU utilization below 15%, the ISR should be kept short. The ISR reads the sensor data, stores it in a buffer, clears the interrupt request and returns control to the main program. Additional processing can be performed later as a background task.

Working Process

Sensor Generates Interrupt
↓
Interrupt Controller Receives Request
↓
Interrupt Vector is Identified
↓
CPU Saves Current Context
↓
Corresponding ISR is Executed
↓
Sensor Data is Read
↓
Interrupt is Cleared
↓
CPU Returns to Main Program

Constraint Satisfaction

| Constraint | Proposed Solution |
| --- | --- |
| CPU utilization below 15% | Short ISR and background processing |
| Interrupt every 1 ms | Fast vectored interrupt handling |
| Vectored interrupts | Separate vector for each sensor |
| Low latency | Memory-mapped I/O |

Conclusion

The proposed system uses vectored interrupts to provide a fast response to multiple sensors. Short interrupt service routines reduce CPU usage, while memory-mapped I/O provides efficient access to sensor registers. Therefore, the design satisfies the real-time requirements of the system.

QUESTION 2: HIGH-SPEED NVMe STORAGE WITH PCIe AND DMA

Given Constraints

Storage throughput must be greater than 3 GB/s.

NVMe over PCIe must be used.

The CPU should not perform bulk data transfers.

DMA and interrupt notification must be included.

Proposed Solution

An NVMe SSD connected through PCIe is used to achieve high-speed storage communication. The system uses DMA to transfer data directly between the NVMe storage device and main memory.

The CPU prepares the required NVMe commands and manages the command queue. However, the CPU does not transfer the complete data directly. The DMA controller performs the bulk transfer.

After the transfer is completed, an interrupt is generated to notify the CPU. The CPU then processes the completion information.

Working Process

CPU Creates NVMe Command
↓
Command Sent Through PCIe
↓
NVMe Controller Receives Command
↓
DMA Transfers Data
↓
Data Stored in Main Memory
↓
Transfer Completed
↓
Interrupt Generated
↓
CPU Handles Completion

Advantages

Provides high-speed data transfer.

Reduces CPU involvement.

Uses PCIe bandwidth efficiently.

DMA transfers large amounts of data directly.

Interrupts notify the CPU after completion.

Constraint Satisfaction

| Constraint | Proposed Solution |
| --- | --- |
| Throughput greater than 3 GB/s | NVMe over PCIe |
| Reduced CPU involvement | DMA controller |
| Transfer completion notification | Interrupt mechanism |
| High-speed storage communication | NVMe SSD |

Conclusion

The combination of NVMe, PCIe, DMA and interrupts provides an efficient high-speed storage system. DMA reduces CPU workload by transferring data directly between storage and memory, helping the system achieve the required high throughput.

QUESTION 3: MULTI-DEVICE COMMUNICATION USING USB AND THUNDERBOLT

Given Constraints

The system contains 6 USB peripherals.

The system contains 2 high-speed Thunderbolt devices.

Interrupt starvation must be avoided.

Both hardware and software interrupts must be supported.

Bus arbitration must provide fair access.

Proposed Solution

A USB controller is used to manage the six USB devices, while a separate Thunderbolt controller handles the two high-speed Thunderbolt devices.

A centralized interrupt controller manages interrupt requests from all connected devices. Hardware interrupts are assigned suitable priority levels based on their importance.

Software interrupts are used for operating-system services and background tasks.

A round-robin arbitration technique is used to provide fair access to the communication bus. This ensures that no single device continuously occupies the bus and prevents other devices from being ignored.

Working Process

Device Generates Request
↓
Interrupt Controller Receives Request
↓
Priority of Request is Checked
↓
CPU Executes Corresponding ISR
↓
Required Data Transfer is Performed
↓
Bus is Released
↓
Next Device Request is Processed

Avoiding Interrupt Starvation

Interrupt starvation can be reduced by:

Assigning appropriate priority levels.

Keeping interrupt service routines short.

Using fair bus arbitration.

Allowing lower-priority devices to receive service.

Moving lengthy operations to background tasks.

Bus Arbitration

The round-robin method provides each requesting device with an opportunity to access the communication resource.

Example sequence:

USB 1 → USB 2 → USB 3 → USB 4 → USB 5 → USB 6 → Thunderbolt 1 → Thunderbolt 2

After completing the sequence, the process can continue again from the first requesting device.

Conclusion

The proposed design supports multiple USB and Thunderbolt devices efficiently. Priority-based interrupt handling ensures fast responses, while round-robin arbitration provides fair access and helps prevent interrupt starvation.

QUESTION 4: DATA ACQUISITION SYSTEM USING DMA

Given Constraints

The system receives a continuous data stream of 500 MB/s.

CPU involvement must be minimal.

DMA and interrupt-driven I/O must be compared.

Device registers must use memory-mapped I/O.

Proposed Solution

DMA is selected as the main data transfer mechanism because the system handles a continuous high-speed data stream of 500 MB/s.

The data acquisition device transfers information directly to main memory using a DMA controller. The CPU does not need to handle every byte or word individually.

The device control and status registers are accessed through memory-mapped I/O.

When a block of data has been transferred successfully, the DMA controller generates an interrupt to notify the CPU.

Working Process

Data Acquisition Device
↓
Memory-Mapped Device Registers
↓
DMA Controller
↓
Main Memory Buffer
↓
Block Transfer Completed
↓
Interrupt Generated
↓
CPU Processes Completed Buffer

Comparison Between DMA and Interrupt-Driven I/O

| Feature | DMA | Interrupt-Driven I/O |
| --- | --- | --- |
| CPU involvement | Very low | High |
| Suitable for 500 MB/s | Yes | Less suitable |
| Data transfer | Directly to memory | CPU handles transfer |
| CPU overhead | Low | High |
| Throughput | High | Comparatively lower |
| Efficiency | High | Lower for continuous data |

Why DMA is Preferred

For a continuous data stream of 500 MB/s, interrupt-driven I/O can create excessive CPU overhead because the processor must respond to many interrupts.

DMA transfers large blocks of data directly into memory. The CPU is interrupted only after a block transfer is completed, allowing it to perform other important operations.

Conclusion

DMA is more suitable for continuous high-speed data acquisition. It provides high throughput, reduces CPU involvement and transfers data efficiently into main memory.

QUESTION 5: NESTED INTERRUPT HANDLING MECHANISM

Given Constraints

The system must support nested interrupts.

Maximum interrupt latency must be less than 10 µs.

High-priority devices must use vectored interrupts.

Software interrupts should be used for background tasks.

Proposed Solution

A priority-based interrupt controller is used to manage interrupt requests. High-priority devices are assigned individual interrupt vectors to allow the CPU to quickly identify the required Interrupt Service Routine.

When an interrupt occurs, the CPU saves its current execution state and executes the corresponding ISR.

If a higher-priority interrupt occurs while another ISR is executing, the current ISR is temporarily suspended. The CPU then services the higher-priority interrupt first. After completing the higher-priority ISR, the processor returns to the previously interrupted ISR.

Software interrupts are used for non-critical background operations and operating-system services.

Nested Interrupt Process

Main Program Execution
↓
Interrupt Occurs
↓
CPU Saves Context
↓
First ISR Starts
↓
Higher-Priority Interrupt Occurs
↓
Current ISR is Suspended
↓
CPU Executes Higher-Priority ISR
↓
Higher-Priority ISR Completes
↓
Return to Previous ISR
↓
Return to Main Program

Methods for Maintaining Low Interrupt Latency

The following techniques help maintain interrupt latency below 10 µs:

Use vectored interrupts.

Keep ISRs short.

Use hardware-based priority selection.

Avoid disabling interrupts for long periods.

Use DMA for large data transfers.

Move time-consuming tasks to background processing.

Interrupt Priority Table

| Priority Level | Type of Operation |
| --- | --- |
| 1 – Highest | Critical real-time device |
| 2 | High-speed I/O device |
| 3 | Normal device |
| 4 – Lowest | Background software interrupt |

Conclusion

A priority-based vectored interrupt system with nested interrupt support provides a fast response to critical events. By keeping ISRs short and allowing higher-priority interrupts to preempt lower-priority operations, the system can achieve low interrupt latency and efficiently handle multiple devices.

OVERALL CONCLUSION

The proposed solutions demonstrate different methods of handling I/O communication under specific system constraints. Vectored interrupts provide quick responses for real-time devices, while NVMe over PCIe with DMA supports high-speed storage transfers with reduced CPU involvement. Round-robin arbitration provides fair communication among multiple devices. DMA is highly suitable for continuous high-speed data acquisition, and nested interrupts ensure that critical events receive immediate attention. Together, these techniques improve the overall efficiency and performance of computer I/O systems.